REPUBLIC OF CAMEROON

*********************************

PEACE-WORK-FATHERLAND

*********************************

UNIVERISTY OF BUEA

*********************************

NAME OF STUDENT     : OBIH DAVID CHIMAOBI

MATRICULATION number : FE22A421

Student contact                     : 679225169

NAME OF INSTRUCTOR       : Dr.  KEMENI VALERY

Final Year project Activity Report

Design of a high Performance Computer

Point Addressing and setting upgrades of service jetson

Jetson Nano Nvidia module

Objectives:

The main objective of this report is to show case the progress of the project and also sight the difficulty and hindrances that push backward the progress of this project and also the aid of the lecturer to provide insight of the projects.

This document brings more light on the hardware, software and program/project development space which a jetson dev module. Nvidia provide a developer space known as jetpack which provide an ubuntu powered OS to project out the activity of the module and the objectives are to see on how to construct plus build out the best result form this module.

Jetson Nvidia has multiple possibility which converges to objective to build a four layer edge computing device that can carry out simultaneous operations and control at high speed , great accuracy and more enhancement.

Title: Full environment installation and control process of the various packages of the Jetpack provision OS.

What is a Jetson nano Module: this is powerful AI development kit that is able to carry out several operations and be built to undergo variety of computing activities. these various activities are;

- Edge computing

- Robotics

- Artificial Intelligence (AI)

- Computer vision

- Embedded systems

- Autonomous devices

- IoT applications

It is essentially a miniature Linux computer equipped with a GPU optimized for AI and parallel computing.

NB: Multiple retailers and variety of companies in partnership with the Nvidia provides the variant of the Jetson nano developer Kit, example:  Okdo

Unlike a normal PC CPU-focused system, the Jetson Nano combines:

- ARM-based CPU

- CUDA-capable NVIDIA GPU

- AI acceleration

- GPIO hardware interfaces

This allows it to process camera feeds, sensors, neural networks, and real-time data directly on the device without relying heavily on cloud servers.

What the Jetson Nano Does

The Jetson Nano acts as an:

AI processing node, Embedded server, Vision processing unit, Robotics controller, Edge inference machine, Distributed computing device.

It runs Linux (Ubuntu-based Jetpack OS) and can execute:

- Python

- C/C++

- CUDA applications

- OpenCV

- PyTorch

- TensorFlow

- ROS (Robot Operating System)

- Docker containers

Main Hardware Components

CPU

Quad-core ARM Cortex-A57 processor.

Handles:

- Operating system

- General computing

- Networking

- Logic and control

GPU

128-core Maxwell GPU.

Handles:

- CUDA acceleration

- AI inference

- Parallel computing

- Image/video processing

This is the most important component for AI workloads.

RAM

Usually:

- 4GB LPDDR4 (classic Nano)

- Some 2GB versions exist

Used for:

- AI models

- OS memory

- Video processing

- Applications

Interfaces

The Jetson Nano includes:

USB ports, HDMI and DisplayPort, CSI camera connector, Ethernet, GPIO pins, I2C, SPI UART, and MicroSD storage.

This allows connection to:

- Cameras

- Sensors

- Motors

- Displays

- Storage devices

- Network switches

- Other Jetsons

What It Can Accomplish

1. AI and Deep Learning

The Nano can run:

- YOLO object detection

- Face recognition

- Image classification

- Speech recognition

- Tracking systems

Example:

- Security surveillance

- Smart cameras

- AI drones

2. Computer Vision

Using:

- OpenCV

- CUDA acceleration

- CSI cameras

It can:

- Detect objects

- Count people

- Track movement

- Analyse traffic

- Read license plates

3. Robotics

The Nano is heavily used in robotics because it can:

- Process sensors

- Control motors

- Run AI navigation

- Handle SLAM

- Communicate with microcontrollers

Examples:

- Autonomous robots

- Warehouse robots

- AI vehicles

4. Edge Computing

Instead of sending data to cloud servers, the Jetson processes data locally.

Advantages:

- Lower latency

- Faster response

- Reduced bandwidth usage

- Better privacy

This is critical for:

- Real-time AI

- Industrial automation

- Smart cities

Cluster / HPC Project Context

Since you are building a multi-node Jetson system, each Nano can become:

- A compute node

- A camera-processing node

- A storage node

- A network controller

- An inference node

You can interconnect them through:

- Ethernet switch

- MQTT

- SSH

- Docker Swarm

- Kubernetes

- MPI

This transforms several small Jetsons into a distributed edge-computing cluster.

What Is Required to Use a Jetson Nano

1. Power Supply

Very important.

Typical requirement:

- 5V / 4A barrel jack supply

Insufficient power causes:

- Random shutdowns

- Boot failures

- USB instability

- AI processing crashes

For ramp/power distribution design:

- Stable regulated 5V rail

- Adequate current capacity

- Cooling consideration

This design is to follow the path to design a cluster of multiple node for edge computing and data processing.

2. Storage

Usually:

- MicroSD card (minimum 32GB recommended)

Used for:

- Operating system

- AI models

- Software

High-end setups may use:

- SSD via USB

3. Operating System

You flash:

- NVIDIA JetPack SDK

JetPack includes:

- Ubuntu Linux

- CUDA

- cuDNN

- TensorRT

- OpenCV support

Official downloads:

- NVIDIA JetPack SDK

- NVIDIA Jetson Downloads Center

Additional Things Commonly Required

Cooling

The Nano gets hot under AI workloads.

Recommended:

- Heat sink

- Active fan

- Proper airflow

For cluster systems:

- Central airflow path

- Rack ventilation

Networking

For a cluster:

- Gigabit Ethernet switch

- Static IP configuration

- SSH enabled

Cameras

Common choices:

- CSI camera modules

- USB webcams

Used for:

- AI vision

- Video analytics

- Robotics

Software Stack

Most Jetson projects install:

- CUDA

- OpenCV

- PyTorch

- TensorFlow

- Docker

- MQTT

- ROS

Practical Real-World Applications

Smart Surveillance

AI cameras with:

- Intrusion detection

- Face recognition

- Tracking

AI Traffic Monitoring

- Vehicle counting

- Plate recognition

- Traffic flow analysis

Edge AI Server

A local AI inference server for:

- Factories

- Schools

- Hospitals

- Smart homes

Mini Supercomputer / HPC Cluster

Multiple Jetsons can:

- Share workloads

- Run distributed AI

- Simulate HPC concepts

- Execute parallel computing tasks

This details and processing ability aligns to the detail and foster a possible design of our AI cluster and possible the discovery of new ways to enhance the science relative to AI.

Limitations of the Jetson Nano

Despite being powerful for embedded AI, it has limits:

- Limited RAM

- Lower CPU performance than desktops

- No high-end GPU like RTX cards

- Limited PCIe expansion

- Thermal constraints

Heavy AI training is difficult.

The Nano is best for:

- AI inference

- Edge computing

- Lightweight distributed computing

Not:

- Massive LLM training

- Large-scale HPC simulation

Why the Jetson Nano Is Important

The Nano bridges:

- Embedded systems

- AI acceleration

- Robotics

- Edge computing

- Distributed systems

It allows you to build:

- Real AI infrastructure

- Low-power clusters

- Intelligent devices

- Research prototypes

- Industrial edge systems

This information above were to sign in the presence of the jetson Nano as an innovative and highly productive device use both in laboratory and also in industrial activities.

Despite the high cost of each Module it provides enhance and more productive aspect in the study of embedded AI and other forms of electronic related projects.

Jetson nano environment build

Jetpack OS provided packages

this is quit fast operation in the recent models of the NVidia dev’s Kit since they are built with the recent modelling chips, memory and also power processing components further the recent LLM (large language Model) processing science as deep learning and supervised learning. the other variant as the Jetson Nano Orin dev’ kit is on other level compared to the jetson nano module since it can be processes with a power supply ~9V/5.1A (45W) power supply and only that is a great difference and let not just speak of its statistic which are way high to that of the Jetson nano module.

Installation of Jetpack OS

To install the operating system on an NVIDIA Jetson module such as the NVIDIA Jetson Nano Developer Kit, you install JetPack SDK, which contains:

- Ubuntu-based Linux OS (Jetson Linux / L4T)

- CUDA

- cuDNN

- TensorRT

- OpenCV support

- AI acceleration libraries

- Drivers and firmware

The installation method depends on your Jetson model. Since your project discussions indicate you are using a standard Jetson Nano module, the easiest method is:

Method 1 — Flash JetPack to microSD Card (Recommended for Jetson Nano Dev Kit)

Requirements

Hardware

- Jetson Nano Developer Kit

- microSD card (minimum 32GB, Class 10/UHS-1 recommended)

- 5V/4A power supply (barrel jack recommended)

- HDMI monitor

- USB keyboard and mouse

- Ethernet cable or Wi-Fi dongle

PC Requirements

A Windows, Linux, or Mac computer for preparing the SD card.

Step 1 — Download JetPack Image

Go to the official NVIDIA Jetson download page:

NVIDIA JetPack Downloads

For Jetson Nano, download:

- Jetson Nano Developer Kit SD Card Image

You may also access Jetson software from:

NVIDIA Jetson Developer Center

Step 2 — Download Balena Etcher

Balena Etcher is used to flash the image to the SD card.

Download:

Balena Etcher

Install it on your PC.

Step 3 — Insert microSD Card into PC

Use:

- SD card slot
or

- USB SD card reader

Step 4 — Flash the Image

Open Balena Etcher.

Process:

- Select downloaded JetPack image (.img or .zip)

- Select microSD card

- Click Flash

Wait until flashing completes.

This may take 5–20 minutes depending on the SD card speed.

Step 5 — Insert SD Card into Jetson Nano

Insert the flashed microSD card into the slot under the board.

Step 6 — Connect Peripherals

Connect:

- HDMI display

- Keyboard

- Mouse

- Ethernet cable (recommended initially)

Step 7 — Connect Power

For stability in cluster or AI workloads, use:

- 5V/4A barrel jack supply

Avoid weak USB phone chargers.

If using barrel jack:

- Place jumper on J48 power pins (important)

Step 8 — First Boot

Power ON the Jetson Nano.

You should see:

- NVIDIA logo

- Ubuntu setup wizard

Setup includes:

- Username

- Password

- Time zone

- Keyboard layout

- Network

Step 9 — Update the System

Open Terminal and run:

Sudo apt update
sudo apt upgrade -y

Then reboot:

sudo reboot

Step 10 — Verify JetPack Installation

Check system info:

cat /etc/nv_tegra_release

Check CUDA:

nvcc --version

Check Python:

python3 --version

Jetson Nano Headless Setup & OpenCV CUDA Installation Report

This operation is long and require a proper search and process control of command lines on the ubuntu Terminal and verify the directory positioning and environment files that are required to position

- CUDA

- OPEN CV

- Build  directory ( -desktop:~/opencv_build/opencv/build)

Summary of operation

Successfully configured an NVIDIA Jetson Nano Developer Kit for headless operation and installed OpenCV 4.5.0 with full CUDA acceleration. The system is now fully accessible via SSH and WiFi, with GPU-accelerated computer vision capabilities verified.

| Component | Specification |
| --- | --- |
| Hardware | NVIDIA Jetson Nano Developer Kit |
| OS | Ubuntu 18.04.6 LTS (L4T 32.7.6) |
| Kernel | 4.9.337-tegra aarch64 |
| CUDA Version | 10.2 |
| cuDNN Version | 8.2.1 |
| Python Version | 3.6.9 |
| OpenCV Version | 4.5.0 (CUDA enabled) |

Headless Configuration

| Task | Status | Details |
| --- | --- | --- |
| WiFi Adapter Setup | Complete | LB-Link MT7601U (148f:7601) |
| WiFi Connection | Complete | Connected to PADDYearpay1 |
| IP Address | Obtained | 192.168.1.25 |
| SSH Server | Running | Port 22 accessible |
| Remote Desktop (xrdp) | Configured | Xfce/LXDE desktop |
| Swap Space | Expanded | 6GB + existing = 7.9GB total |

Network Configuration:
Interface: wlan0

IPv4: 192.168.1.25/24

Gateway: 192.168.1.1

Status: Connected

They are various way to undergo headless mode on a jetson module the one mostly common ( GUI mode) using Xrdp or VNC but those methods are going to be expanded later in other documentation.

System Updates & Fixes

| Issue | Resolution |
| --- | --- |
| NVIDIA bootloader configuration error | Resolved with post-install script modifications |
| xrdp session crash | Switched from GNOME to Xfce/LXDE desktop |
| apt lock conflicts | Cleared lock files and killed stuck processes |
| OpenCV compilation CUDA error (cuda/std/tuple) | Downgraded from 4.10.0 to 4.5.0 |

OpenCV 4.5.0 with CUDA Installation

Command lines

# 5. Download OpenCV 4.5.0 (Crucial for Jetson Nano)

mkdir -p ~/opencv_build && cd ~/opencv_build

wget -O opencv.zip https://github.com/opencv/opencv/archive/4.5.0.zip

wget -O opencv_contrib.zip https://github.com/opencv/opencv_contrib/archive/4.5.0.zip

unzip opencv.zip && unzip opencv_contrib.zip

mv opencv-4.5.0 opencv && mv opencv_contrib-4.5.0 opencv_contrib

rm opencv.zip opencv_contrib.zip

# 6. CMake Configuration (CUDA Architecture 5.3 is critical for Nano)

cd ~/opencv_build/opencv

mkdir -p build && cd build

cmake -D CMAKE_BUILD_TYPE=RELEASE \

-D CMAKE_INSTALL_PREFIX=/usr/local \

-D WITH_CUDA=ON \

-D WITH_CUDNN=ON \

-D CUDA_ARCH_BIN=5.3 \

-D CUDA_ARCH_PTX="" \

-D ENABLE_FAST_MATH=ON \

-D CUDA_FAST_MATH=ON \

-D WITH_CUBLAS=ON \

-D BUILD_opencv_python3=ON \

-D BUILD_opencv_python2=OFF \

-D OPENCV_EXTRA_MODULES_PATH=~/opencv_build/opencv_contrib/modules \

-D PYTHON3_EXECUTABLE=$(which python3) \

-D PYTHON3_NUMPY_INCLUDE_DIRS=$(python3 -c "import numpy; print(numpy.get_include())") \

-D BUILD_EXAMPLES=OFF \

-D BUILD_TESTS=OFF ..

# 7. Compile and Install (Takes 2-4 hours)

make -j4

sudo make install

sudo ldconfig

# 8. Fix potential NVIDIA linker issue

sudo sh -c "echo '/usr/local/cuda/lib64' >> /etc/ld.so.conf.d/nvidia-tegra.conf"

sudo ldconfig

Verification & Cleanup

# 9. Verify Installation

python3 -c "import cv2; print('OpenCV:', cv2.__version__)"

python3 -c "import cv2; print('CUDA Devices:', cv2.cuda.getCudaEnabledDeviceCount())"

python3 -c "print(cv2.getBuildInformation())" | grep "CUDA"

# 10. Remove Build Directory to save space (Optional)

rm -rf ~/opencv_build

# 11. Disable Swap (Optional, to save SD card wear)

sudo swapoff /swapfile

sudo rm /swapfile

Final Summary of Achievements

At this level

- Headless Operation: The Jetson Nano is fully operational without a monitor, controlled via SSH (192.168.1.25) and Remote Desktop (xrdp).

- WiFi Connectivity: Successfully configured the LB-Link MT7601U adapter (ID 148f:7601).

- OpenCV 4.5.0 with CUDA: Successfully compiled and installed OpenCV 4.5.0 from source. This specific version is the most stable release compatible with the Jetson Nano's final JetPack version .

- GPU Acceleration: Verified that OpenCV can access the Jetson Nano's CUDA cores, enabling hardware acceleration for computer vision tasks.

- Environmental Stability: The system is locked into the final supported configuration by NVIDIA (JetPack 4.6 / CUDA 10.2), which is a mature and highly stable environment