Sujestive control process for the alarm service

For your Jetson cluster project (ESP32 sensors → Node 1 → Node 2 → Node 3 → AI Engine), I would recommend thinking of the alarm system in 4 layers.

Layer 1: Sensor Layer (ESP32)

The ESP32 should not make critical decisions.

Its role is only:

- Read Temperature

- Read Pressure

- Read Humidity

- Read Sensor Health

- Send data every few seconds

Example:

| Parameter | Value |
| --- | --- |
| Temperature | 48°C |
| Pressure | 1.3 bar |
| Humidity | 76% |
| Timestamp | 12:05:23 |

The ESP32 can activate a local buzzer only if communication with the cluster is lost.

Layer 2: Analysis Layer (Jetson Nodes)

The AI should never react to a single reading.

For example:

Current temperature:

44°C
45°C
46°C
47°C
48°C

This indicates a rising trend.

But:

40°C
41°C
65°C
40°C
41°C

is likely a spike or sensor error.

The AI should evaluate:

Trend

Is the value increasing?

Duration

How long has it remained abnormal?

Correlation

Are multiple sensors indicating danger?

Example:

| Temp | Pressure | Humidity |
| --- | --- | --- |
| High | Normal | Normal |

Possible fan issue.

| Temp | High | High |
| --- | --- | --- |
| High | High | Normal |

Possible overheating vessel.

| Temp | Pressure | Humidity |
| --- | --- | --- |
| High | High | High |

Possible critical event.

Layer 3: Risk Classification

Instead of:

SAFE
DANGER

use levels.

Level 0

Normal

No alarm

Level 1

Warning

Slight abnormality

Example:

Temp > 50°C for 2 minutes

Level 2

Alert

Significant abnormality

Example:

Temp > 60°C
Pressure rising

Level 3

Critical

Immediate attention required

Example:

Temp > 75°C
Pressure > threshold
Humidity abnormal

Level 4

Emergency

Equipment damage likely

Example:

Temp continuously rising
Pressure critical
AI predicts failure

Layer 4: Alarm Intelligence

This is where your idea becomes interesting.

Different patterns identify different problems.

Instead of one continuous beep:

Temperature Alarm

Long pulses

BEEEEEP
pause
BEEEEEP
pause

Pattern:

1 second ON
1 second OFF

Meaning:

"Heat issue"

Pressure Alarm

Fast pulses

BEEP BEEP BEEP
pause
BEEP BEEP BEEP

Pattern:

0.2s ON
0.2s OFF

Meaning:

"Pressure issue"

Humidity Alarm

Double pulse

BEEP-BEEP
pause
BEEP-BEEP

Meaning:

"Humidity issue"

Multi-Sensor Critical Event

Alternating pattern

BEEP
BEEP-BEEP
BEEEEEEP

repeating

This immediately tells the operator:

More than one variable is dangerous.

AI-Based Decision Example

Imagine:

Temperature = 62°C
Pressure = Normal
Humidity = Normal

AI Response:

Risk = Warning
Alarm = Temperature Pattern

After 5 minutes:

Temperature = 67°C
Pressure = Rising
Humidity = Normal

AI Response:

Risk = Alert
Alarm = Temperature + Pressure Pattern

After 10 minutes:

Temperature = 74°C
Pressure = High
Humidity = High

AI Response:

Risk = Critical
Alarm = Emergency Pattern
Notify Dashboard
Log Event

Future Improvement: Predictive Alarm

This is where the Jetson cluster becomes valuable.

Instead of:

Alarm because temperature reached 80°C

the AI predicts:

Based on the last 10 minutes, temperature will reach 80°C in 3 minutes.

The alarm activates early.

This is essentially predictive maintenance, which is used in industrial systems, power plants, and data centers.

Proposed Architecture

ESP32 Sensors
      │
      ▼
Node 1
(Data Collection)
      │
      ▼
Node 2
(Data Processing)
      │
      ▼
Node 3
(Alarm Controller)
      │
      ▼
Buzzer + Dashboard
      │
      ▼
AI Engine
(Decision Making & Prediction)